APPENDIX 2: Motomed Letto 2 Cycling Protocol

Cycling will be performed for 45 minutes per session using the Motomed Letto 2 cycle.

How to Detect Active Cycling (all criteria are required to occur simultaneously):

- Active cycling time will be displayed on screen

- While patient is actively cycling, there is a moving bike icon on the screen

- While patient is actively cycling, actual cycling speed displayed on screen is greater than the default setting of 10 or 25 rpm

Training will occur in 3 Phases according to the 3 Steps below:

I: ASSESSMENT PHASE (10 minutes):

- Using the following Initial Settings:

Spasm control: off; Default speed: 10 rpm

- Initial gear setting: 0 (this is default gear)

- Vigorously verbally encourage the patient to cycle – the goal is to maximize the duration that the patient spends actively cycling.

- At the end of 10 minute interval, determine the duration of active cycling

- if >5 minutes, move to Active Interval 1 [Step IIA (2)].

- if <5 minutes, move to Assisted Interval 1 [Step IIB (2)].

IIA: ACTIVE INTERVALS (3 intervals of 10 minutes each; 30 minutes total):

- Vigorously verbally encourage the patient to actively cycle during all intervals.

- Active Interval 1  (10 minutes): Select the gear

- Start at Gear 1 If today is the first day of the intervention    OR

- Start at Gear 1 If the highest gear achieved for >5 minutes in one interval on the prior day’s intervention was <2    OR

- Start at one gear lower than the highest gear achieved for >5 minutes in one interval on the prior day’s intervention when the highest gear achieved was >3  (i.e. Yesterday patient cycled at gear 5 for 6 minutes, today patient starts at gear 4)

- At the end of 10 minute interval, determine duration of active cycling during this interval using this calculation:

Cumulative Active Cycling Minutes at 	MINUS		Cumulative Active Cycling Minutes

end of current interval   					at end of prior interval

- If the duration of active cycling is:

- >5 minutes, move to Active Interval 2 [Step IIA (5)].

- <5 minutes, move to Assisted Interval 2-A [Step IIB (3)].

- Active Interval 2 (10 minutes): Increase the Gear by 1 for and repeat step (3). If the duration of active cycling is:

- >5 minutes, move to Active Interval 3-A  [Step IIA (6)],

- <5 minutes, move to Assisted Interval 3-A [Step IIB (5)]

- Active Interval 3A (10 minutes): Increase the Gear by 1 and repeat step (3).

- Active Interval 3B (10 minutes): Select the gear

- Start at Gear 1 If today is the first day of the intervention    OR

- Start at Gear 1 If the highest gear achieved for >5 minutes in one interval on the prior day’s intervention was <2    OR

- Start at one gear lower than the highest gear achieved for >5 minutes in one interval on the prior day’s intervention when the highest gear achieved was >3  (i.e. Yesterday patient cycled at gear 5 for 6 minutes, today patient starts at gear 4)

IIB: ASSISTED INTERVALS (3 intervals of 10 minutes each; 30 minutes total):

- Vigorously verbally encourage the patient to actively cycle during all intervals

- Assisted Interval 1 (10 minutes):

- Increase speed to 25 RPM with Gear set at 0

- Do not assess for active cycling

- Move to Assisted Interval 2-B [Step IIB (5)]

- Assisted Interval 2-A (10 minutes):

  - Keep speed at 10 RPM

  - Decrease gear by 1.

- Determine duration of active cycling [as defined in Step IIA (3) above]. If duration of active cycling is:

    - >5 minutes, move to Active Interval 3-A [Step IIA (6)]

    - <5 minutes, move to Assisted Interval 3 A [Step IIB (6)]

- Assisted Interval 2-B (10 minutes):

- Decrease speed to 10 RPM

- Keep gear at 0.

- Determine duration of active cycling [as defined in Step IIA (3) above]. If duration of active cycling is:

    - >5  minutes, move to Active Interval 3-B [Step IIA (7)]

    - <5 minutes, move to Assisted Interval 3B [Step IIB (7)]

- Assisted Interval 3A  (10 minutes):

  - Decrease Gear by 1

  - If already at Gear 0, then increase speed to 25 RPM

- Assisted Interval 3B (10 minutes):

  - Increase speed to 25 RPM with Gear set at 0.

III: COOL DOWN Phase (5 minutes):

  - Revert to Initial Settings (spasm control off; 10 RPM, gear set at 0).

  - Encourage slow-paced active cycling.

Assessment Phase

- Gear 0

- Speed: 10 RPM

- 10 minutes

Assessment Phase

- Gear 0

- Speed: 10 RPM

- 10 minutes

Vigorously verbally encourage the patient to actively cycle during each interval

Vigorously verbally encourage the patient to actively cycle during each interval

Active Cycling >5 minutes?

minutes?

NO

NO

YES

YES

Assisted Interval 1

- Gear 0

- Speed: Change to 25 RPM

- 10 minutes

10 minutes

Assisted Interval 1

- Gear 0

- Speed: Change to 25 RPM

- 10 minutes

10 minutes

Active Interval 1

- Gear: See below for Starting Gear*

- 10 minutes

Active Interval 1

- Gear: See below for Starting Gear*

- 10 minutes

Active Cycling >5 minutes?

minutes?

Active Cycling >5 minutes?

minutes?

NO

NO

Do not assess for active cycling

minutes?

Do not assess for active cycling

minutes?

YES

YES

Assisted Interval 2-B

- Gear 0

- Speed: Change to 10 RPM

- 10 minutes

10 minutes

Assisted Interval 2-B

- Gear 0

- Speed: Change to 10 RPM

- 10 minutes

10 minutes

Active Interval 2

- Gear: Increase Gear setting by 1

- 10 minutes

Active Interval 2

- Gear: Increase Gear setting by 1

- 10 minutes

Assisted Interval 2-A

- Gear: Decrease Gear setting by 1

- 10 minutes

Assisted Interval 2-A

- Gear: Decrease Gear setting by 1

- 10 minutes

Active Cycling>5  minutes?

minutes?

Active Cycling>5  minutes?

minutes?

Active Cycling >5  minutes?

minutes?

Active Cycling >5  minutes?

minutes?

Active Cycling >5 minutes?

minutes?

Active Cycling >5 minutes?

minutes?

NO

NO

NO

NO

NO

NO

YES

YES

YES

YES

YES

YES

Assisted Interval 3-B

- Gear 0

- Speed: Change to 25 RPM

- 10 minutes

Assisted Interval 3-B

- Gear 0

- Speed: Change to 25 RPM

- 10 minutes

Active Interval 3-A

- Gear: Increase by 1

- 10 minutes

Active Interval 3-A

- Gear: Increase by 1

- 10 minutes

Active Interval 3-B

- Gear: See below for Starting Gear*

- 10 minutes

Active Interval 3-B

- Gear: See below for Starting Gear*

- 10 minutes

Assisted Interval 3-A

- Gear: Decrease by 1

- If already at Gear 0, Change to 25 RPM

- 10 minutes

Assisted Interval 3-A

- Gear: Decrease by 1

- If already at Gear 0, Change to 25 RPM

- 10 minutes

Cool Down Interval

- Gear 0

- Speed: 10 RPM

- 5 minutes

Cool Down Interval

- Gear 0

- Speed: 10 RPM

- 5 minutes

*Starting Gear:

If today is the first day of the intervention, use Gear 1; Otherwise:

| Highest gear achieved >5 min. during a completed 10 min. active interval on prior day | Starting gear |
| --- | --- |
| < 2 | 1 |
| > 3 | 1 gear lower than highest gear achieved** |

(i.e. **Yesterday patient cycled at gear 5 for 6  minutes, today patient starts at gear 4)

| SAFETY TO COMMENCE / CONTINUE IN-BED CYCLE ERGOMETRY EXERCISE SESSION **If any condition below is present for >15 minutes duration within the 2 hours prior to a cycling session, the session should not be started at that time. If any condition, below, occurs during cycling session, the session should be stopped. |  |
| --- | --- |
| Heart Rate | 1= HR <50 beats/minute 2= HR >140 beats/minute 3= And/or develops new arrhythmia (including new onset atrial fibrillation) |
| Other Cardiovascular | 1= New onset chest pain of potential cardiac origin 2= Presence of extracorporeal membrane oxygenation (ECMO) or intra- aortic balloon pump (IABP) |
| Blood Pressure | 1= MAP <65 mmHg or below target MAP level (whichever is lower) 2= MAP >120 mmHg or above target MAP level (whichever is higher) 4= Patient requires 1 vasopressor at >50% of maximum dose or ≥2 vasopressors at 40% of maximal dose 5= A clinically important increase in vasopressor dose 6= Patient becomes pale/sweaty or requests to stop due to feeling unwell |
| Respiratory | 1= Fi02 >0.8 2= PEEP >15 cmH20 3= SpO2 falls >10% below resting level or <85% of target level for >60 seconds |
| Other Reason | 1= New diagnosis of a pulmonary embolus or deep vein thrombosis in leg without therapeutical anti-coagulation for >36 hours 2= Assessed by medical staff as approaching imminent death or withdrawal of medical treatment 3= Temperature >41˚C or <34˚C 4= Muscle inflammation (e.g., rhabdomyolysis, myositis, neuroleptic malignant syndrome, or serotonin syndrome) 5= Pressure ulcer (grade 3 or 4) in body region in contact with cycle 6= Lactate >3.0 mmol/L that is unexplained and not decreasing 7= Creatine kinase >400 U/L that is unexplained and not decreasing 8= Platelets <20,000/mm3 unless approved by ICU medical staff 9= Infusion or bolus dosing of neuromuscular blocking agent |